Supersilent PA rider 2003.

PA:

Prefered systems: D&B, EAW, Meyer Sound. Powerful, linear, including 

sufficient sub bass. Free from noize, buzz, hiss.

Monitors:

4 pcs. (one for each musician): powerful & linear, on separate feeds 

from the PA mixer, or (preferably) from a separate on-stage monitor

mixer.

PA mixer:

The mixer must be in mint condition, without scratchy faders, and with 

basic eq facilities and preferably with sweepable highpass filters.

Prefered mixer: Midas Heritage or Legend.

Stage lines:

1
kick

- Beyer M88

2
kick

- Beyer M88

3
snare

- Shure SM 57

4
hihat

- Shure SM 57

5
floor tom
- Shure SM 57

6
medium tom
- Shure SM 57

7
medium tom
- Shure SM 57

8
small tom
- Shure SM 57

9
(spare percussion condencer mic.) 


10
(spare percussion condencer mic.)

11
OH - L

- Neumann KM 140/143/184

12
OH - R

- Neumann KM 140/143/184

13-14
DI (line level) on left side of stage. Staale Storloekken.

15
Shure SM 58 on left side of stage (on a small stand, with boom). 

Staale's keyboard amp.

16
Shure SM 58 in centre of stage (on a small stand, with boom. Arve 

Clean trumpet.

+!
Shure SM 58 in centre of stage (on a small stand, with boom) - Not 

connected to PA. Arve FX trumpet.

17-18
DI (line level) in centre of stage. Arve Henriksen.

19-20
DI (line level) on right side of stage. Helge Sten.

21-23
(Spare DI (line level) on right side of stage.)

(24)
(Shure SM 58 located at mixer position, for communication and PA 

line up)

(25-30)
(Compressor returns)

(31-32)
(reverb return)


Effects:

One Lexicon reverb unit.

Gates on ch. 1 and 2.

Stereo compressors on group 1/2, 3/4, 5/6. Preferably connected 

between group outputs (not inserts) and channel inputs, so they can be 

independently routed and eq'd.

If possible (less important): expander/compressors on ch. 13/14, 16, 

17/18, 19/20.

PA and all monitor feeds must have 1/3 octave graphic eq's.

Please let me know in advance who will be working as stage manager and 

system engineer.

BACKLINE 

Stage Left Ståle Storløkken

1x Fender Rhodes piano

1x Fender Twin amp/ or Marshall valve combo

1x Yamaha DX 7 Synthesizer

1x Sequential Circuits Prophet Synthesizer

Stage right Helge

1x Standard 5 octave midi keyboard (Yamaha DX7 or similar)

Stage Center (Jarle Vespestad)drums

2x Bassdrums with 2 pedals,1x Snare drum with stand,as many toms as you got(!) with hardware,1x Drum Chair,1x Hi-hat Stand,3x Cymbal Stands.

FOH engineer: Audun Strype. Tollbugt. 28 0157 Oslo. Phone +47 

22425870. Fax +47 22411746. www.strype.com audun@strype.com





Sign promoter 








Sign . promoter promoter








